# Nuplex 80+ Advanced Control Complex (ACC)

## Summary

Nuplex 80+ is the Advanced Control Complex (ACC) developed by Combustion Engineering (later ABB-CE, then Westinghouse) for the System 80+ standard PWR design, which received NRC final design approval in 1994 and design certification in 1997 (10 CFR Part 52, Appendix B). It was a milestone in advanced-control-room design: a fully integrated, computer-based control room with workstation-style operator consoles, a Critical Function Monitoring System, the Discrete Indication and Alarm System (DIAS), the Data Processing System (DPS) for non-safety, and a separate Class 1E plant-protection system. The architecture and HFE concepts of Nuplex 80+ heavily influenced subsequent Westinghouse / KEPCO designs, including APR1400 (whose MMIS lineage descends from System 80+ via OPR-1000).

## Public sources

| Source | Reference | Repository |
| --- | --- | --- |
| US NRC | CESSAR-DC (Combustion Engineering Standard Safety Analysis Report — Design Certification) for System 80+. Public Tier 2 chapters on ADAMS. Chapter 7 is the I&C / Nuplex 80+ chapter; Chapter 18 is HFE | NRC ADAMS, CESSAR-DC |
| US NRC | System 80+ Final Design Approval (FDA) and Design Certification rulemaking record | NRC ADAMS |
| OECD/NEA | Reports on advanced control rooms; System 80+ is a recurring case study | oecd-nea.org |
| Conference papers | NPIC&HMIT and ANS Annual proceedings 1990s–2000s on System 80+ | IEEE Xplore, OSTI.gov |
| KEPCO / KHNP | APR1400 public documentation (MMIS heritage traces to Nuplex 80+) | NRC ADAMS APR1400 DCD |

## Architectural concepts described in public material

The Tier 2 chapters of CESSAR-DC describe these openly.

- Plant Protection System (PPS) — Class 1E, four channels, 2-of-4 coincidence, separate from non-1E control.

- Engineered Safety Features Component Control System (ESF-CCS) — the actuation side of the safety architecture (where ECCS actuation logic lives in System 80+).

- Data Processing System (DPS) — non-safety, integrates plant data for the operator workstations.

- Discrete Indication and Alarm System (DIAS) — diverse, simpler indication path independent of the DPS, providing a backup for safety-critical indication and alarms.

- Critical Function Monitoring System (CFMS) — operator-aid that tracks the safety functions (reactivity control, RCS inventory/pressure, core heat removal, RCS heat removal, containment integrity, vital auxiliaries).

- Operator workstations and Compact Workstation Panel (CWP) — soft controls plus a small set of dedicated hard controls for the most safety-critical actions.

- Strong functional separation between safety and non-safety, with hardware-enforced isolation across the boundary.

## Information not publicly available

- Application source / control logic / function-block libraries for any specific implementation.

- Proprietary "Tier 1" portions of the design certification.

- Detailed HFE V&V data.