# TELEPERM XS (TXS)

## Summary

TELEPERM XS is a digital safety I&C platform originally developed by Siemens KWU in the 1990s, transferred through Siemens / Areva NP to today's owner Framatome. It is purpose-built for Category A (Class 1E equivalent) protection-system functions — reactor protection, ESFAS actuation, post-accident monitoring — and is qualified under the IEC 61513 / 60880 framework. It has been reviewed by multiple regulators including the US NRC, UK ONR, French ASNR, German GRS, and Finnish STUK. It is engineered using a dedicated tool chain (function-block / SPACE engineering tool, SIVAT off-line validator) — not generic Siemens Step 7 / TIA Portal.

## Public sources

Public references are retrievable from the canonical repositories below. Non-proprietary "(NP)" variants are the ones intended for public release.

| Source | Reference | Repository |
| --- | --- | --- |
| Framatome | TELEPERM XS platform brochure / product overview | framatome.com (Products → I&C) |
| US NRC | Topical Report EMF-2110(NP)-A, TELEPERM XS: A Digital Reactor Protection System, and the associated Safety Evaluation | NRC ADAMS, full-text search |
| US NRC | Plant-specific Safety Evaluation Reports for TXS upgrades (e.g. Oconee RPS replacement) | NRC ADAMS |
| IAEA | TECDOC series on digital I&C modernisation | iaea.org publications, IAEA INIS |
| GRS / RSK (Germany) | German review opinions (in German) | grs.de, rskonline.de |
| ASNR / IRSN (France) | French regulator opinions on EPR I&C (TXS used for Class 1) | asnr.fr |
| Conference papers | NPIC&HMIT, ANS Annual, ENC, IAEA TM-Köln series | OSTI.gov, IEEE Xplore, IAEA INIS |

## Architectural concepts described in public material

These are described at a conceptual level in conference papers and the NRC topical report.

- Function Diagram (FD) engineering — safety logic is captured in a graphical function-block language; the compiler chain produces qualified executable code on the TXS computer.

- SIVAT — an off-line simulator that runs the same compiled application against test vectors prior to plant deployment.

- Hard real-time, fully deterministic cyclic execution.

- Redundancy and division separation — typically four independent divisions for protection functions, with hard isolation between them.

- Communication via the Profibus-derived TXS internal bus for intra-system data, and qualified gateways for any data leaving the safety perimeter.

- Defence-in-depth and diversity with a separate diverse system for ATWS coverage on many installations.

## Information not publicly available

- Application source / function-block libraries used at any specific plant.

- Compiler internals and qualification arguments at the code level.

- Detailed cybersecurity dossier.

- Material falling under NSG Part 1 / Part 2 export controls.