# Emergency Core Cooling System (ECCS)

## Summary

ECCS is a safety function, not a product. It is the set of subsystems that inject coolant into the reactor coolant system after a loss-of-coolant accident (LOCA) so that the fuel cladding stays below the regulatory peak-cladding-temperature limit and the core remains in a coolable geometry. In a typical PWR, ECCS comprises high-pressure safety injection (HPSI), low-pressure safety injection (LPSI), accumulators / safety injection tanks, and long-term recirculation through the containment sump. In a BWR it includes HPCI/HPCS, LPCI/LPCS, ADS, RCIC and the RHR loops. Different reactor designs allocate the function differently. From an I&C perspective, the relevant logic is the detection of initiating events and actuation of ECCS pumps and valves, implemented as part of the Engineered Safety Features Actuation System (ESFAS), which sits in the same Class 1E protection envelope as the reactor protection system.

## Public sources

| Source | Reference | Repository |
| --- | --- | --- |
| US NRC | 10 CFR 50.46 and Appendix K to 10 CFR 50 — legal performance criteria for ECCS | nrc.gov electronic CFR |
| US NRC | NUREG-0800 Standard Review Plan, Chapter 6.3 (ECCS) and Chapter 7.3 (ESFAS) | NRC ADAMS / NUREG library |
| US NRC | Final Safety Analysis Reports (FSAR) Chapter 6.3 of US plants — many are public on ADAMS | NRC ADAMS |
| IAEA | SSR-2/1 Rev.1 (design safety requirements) — chapters on emergency core cooling | iaea.org Safety Standards |
| OECD/NEA | Reports on LOCA phenomena and ECCS performance | oecd-nea.org |
| Vendor design control documents | AP1000 DCD Chapter 6.3, US-EPR FSAR Chapter 6.3 — public Tier 2 material is detailed | NRC ADAMS |

## Architectural concepts (I&C side)

What gets engineered for ECCS is the actuation logic, which is structurally similar to reactor protection but on a different set of plant variables.

- Initiating signals: low pressuriser pressure, high containment pressure, low-low steam-generator level, manual actuation, and combinations of these.

- Two-out-of-three or two-out-of-four coincidence depending on the design and the parameter.

- Master / slave actuation pattern — master logic decides "actuate ECCS"; slave logic per train commands the specific pumps, valves, dampers.

- Train independence — typically two or three fully independent ECCS trains, each fed by its own division of the safety I&C and its own Class 1E power.

- Manual capability on the safety side, plus diverse manual capability for ATWS / cyber-security defence-in-depth.

- Post-accident monitoring (PAM) variables provided by Class 1E instrumentation, qualified per Reg. Guide 1.97 / IEEE 497.

- Sequence-of-events recording with timestamps adequate for post-event reconstruction.